| Western Tooling Services |  |  |  |
| --- | --- | --- | --- |
| 247a Redcatch Road Knowle Bristol BS4 2HQ |  |  |  |
|  |  | www.western-tooling.co.uk |  |

Wintrace Upgrade for Talysurf* 10

Surface texture measurement has changed dramatically in the last few years.  New instruments have appeared which have complex data processing facilities.  Although the basic measurement techniques have remained the same, the latest instruments automatically compute a wide range of numerical parameters, and also offer better filtering of data to remove unwanted surface anomalies.  Unfortunately, such progress always has a price, and the replacement of an older machine which is still working well can be very expensive - and is not always necessary.

We offer a cost effective alternative, which is also more environmentally sound.  Instead of scrapping a basically sound machine, we replace the obsolete electronics and pen recorder with a small modern interface unit, and connect it to a modern desktop computer with a full size plain paper printer.  With our latest software running under Microsoft Windows, you can have a first class instrument with all the analysis capabilities of a brand new machine .  This is a summary of the main features:

Fast Easy Operation:      	Single button measure – no time wasting menus.

Digital Data Filtering:    	Software emulation of DSP.              (ISO 11562:1996)

Filter mode:                    	Roughness, waviness, and unfiltered (profile).

Cut off selection:  	Standard:   8,  2.5,  0.8,  0.25,  0.08 mm (or imperial)

Non standard:   User selectable 8 - 0.08 mm (0.3 - 0.003 inch)

Roughness Parameters:   	Ra,  Rq,  Rp,  Rv,  Rt,  Rsk,  Rti,  Rku,  RLambdaQ,  RDeltaQ,

Ry, Rz (DIN), Rz(ISO)			(ISO 4287:1997)

Waviness Parameters:     	Wa,  Wq,  Wp,  Wv,  Wt,  Wsk,  Wti,  Wku, WLambdaQ,

WDeltaQ, Wy, Wz			(ISO 4287:1997)

** 	Motif Parameters:        	R, AR, W, AW, Rx, Wx               	(ISO12085:1996)

**	Rk Parameters: 		Rpk, Rvk, Mr1, Mr2		       	(ISO 13565: 1996)

Unfiltered Parameters:   	Pa,  Pq,  Pp,  Pv,  Pt,  Psk,  Pti,  Pku,  PLambdaQ, PDeltaQ

Graphical displays:	      	Profile, Amplitude Distribution, and Material Ratio.

Multiple zoom facility     	‘Home in’on selected portions of the profile graph

Metric/imperial option.  	Automatic re-calculation.

Spot and step height       	A useful feature enabling simple measurements from

profile graph using a ‘point and click’ technique.

Visual Setting-up aid:    	Fast component levelling with this easy-to-use aid.

Context sensitive help    	Windows help file facility

File catalogue facility     	Allows stored measurement to be found easily.

10 Measurement Cache 	Last ten measurements stored even if not filed.

Full Colour Profiles       	High clarity screen displays and printouts, with option

for user to select display colours.

Column interlock:		Safety feature to prevent damage to gauge.

*Talysurf is a registered trademark of Taylor Hobson Limited.

**These parameters are available as extensions to the standard software.

Western Tooling Services reserve the right to amend this description in the light of our policy of continuous improvement